Day: 06/07/02

Latitude: 27°37°95 N

Water depth: 615 m

Core number: MD02- 2541

GMT time: 21 :37.95 N

Longitude: 92°12.74 W

Location: Tunica Mounds

Corer type: Calypso core

Corer length: 45.30 m

Core length: 35.34 m

Observations

Corer condition :

Corer bent at 10m

Core condition :

Sections and Sampling

Number of sections recovered and conditions :

24 sections recovered, the last section measures 84 cm, good conditions.

A section of 10 cm was taken at the bottom of every 1.5 m section, 2 10 cm section were taken on section I, VII, and X.

Onboard sampling and post cruise processing :

- Both the working and archive halves will be stored at Texas A&M University

- the 10 cm sections were taken for pore water analysis, ( microbiology, and SO4-, Cl-, CH4, 14C, DIC) (MBARI, Univ.Tokyo)

- several bags were recovered : 1 core catcher, 1 core top

- MST magnetic susceptibility, gamma ray attenuation and p-wave measurements were made downcore, at a resolution of 2cm. Colour measurements were taken every 2 cm.

- USGS core measurements and activities : Elec. Resist ; Vane Shear , water content, (USGS-WH), Headspace Gas (USGS-Menlo Park), Thermal conductivity ( Univ.Victoria)

## Summary of physical and sedimentological observations

Dominant lithologies: interval 0-4.5m consists of greenish gray to dark greenish gray silty clay with abundant foraminifera (upper 2.2m) to greenish gray to dark greenish gray silty clay; interval 4.5-35.34m consists of dark gray clay with foraminifera, in most parts with faint layering; bioturbated.

Minor lithologies: black organic-rich streaks common, several pyrite concretions